Collaborative robotics

AA 2025-2026

BRICK BREAKER REPORT

Authors list: Jessica Dichakdjian, Alberto Caramanna,

Mateo Zeneli, Rebecca Papa e Daniela Macaya

This is a project report template. We here suggest the contents that should be outlined, described and discussed in your report. Indeed, you can include additional section, re-arrange them following the story you want to tell, etc. Please just pay attention in including the design hypothesis with respect to the topics studied during the course.

BEFORE THE EXAM (one week before the examination day, exact date will be communicated close to exam calls) each group has to upload in the dedicated we-beep folder:

- the software developed during the project labs (es. Arduino code);

- a short report of the project following the available template (max 12 pages excluded abstract and index, template font).

EXAM:

•	20 minutes: Discuss the results with the support of a PowerPoint presentation (group work).

•	Present a working demo of the developed project (group work).

•	Take part in a short discussion about the project (individual).

•	Answer questions on the overall course content (individual).

Abstract and graphical abstract

Summarize here you designed solution [Max 300 words].

NB. By reading this paragraph the reader should be able to understand what your system is able to do.

- Index

1.	Motivation of the project [Half page]	3

2.	Objective of the work [Half page]	4

3.	Requirements [One page]	5

3.1 Functional requirements: Report functional requirements along with the corresponding means of verification (see LEZ01 for examples).	5

3.2 Non-functional requirements	6

4.	Materials and methods	6

4.1 Hardware [Half page]	6

4.2 Kinematic and dynamic model [One page]	7

4.3 Control design [Three pages]	8

Hardware level of compliance (LEZ03)	8

Low-level control solution (LEZ04)	8

High-level control solution (LEZ05)	9

Intention detection strategy (LEZ06)	9

4.4 Protocol for testing and validation (LEZ07) [One page]	9

5.	Results [Three pages]	9

6.	Discussion and conclusion [One page]	9

7.	References [Half page]	9

# Motivation of the project [Half page]

Describe the background within which your project is placed (e.g., impact of the reference disease, need of rehab/assistance, etc.)

Among non-communicable diseases (NCDs), stroke remains one of the most significant global health challenges, ranking as the second leading cause of death and the third leading cause of death and disability combined, as measured by disability-adjusted life years (DALYs) worldwide [1]. Beyond its high mortality, stroke represents a major cause of long-term disability, with millions of survivors facing persistent functional limitations that severely reduce their autonomy in activities of daily living.

The long-term disability associated with stroke is largely driven by motor impairments resulting from both primary and secondary damage following the acute event. Lesions affecting descending neural pathways lead to altered neuromotor control, accompanied by structural and functional changes in muscle tissue [2]. Over time, these impairments may progressively worsen, forcing patients to develop compensatory strategies and adaptations that further impact their quality of life.

Stroke survivors experience a wide range of complications, and recovery outcomes vary considerably among individuals. However, extensive research has shown that patients who engage in structured and focused stroke rehabilitation programs achieve significantly better functional outcomes than those who do not receive rehabilitation. For this reason, stroke rehabilitation is strongly recommended for all affected individuals. Rehabilitation strategies are primarily based on targeted, repetitive, and task-specific movements aimed at promoting neuroplasticity and functional recovery.

In recent years, technology-assisted rehabilitation has gained increasing relevance. In particular, robotic-based therapies have emerged as a promising tool to support physical rehabilitation. Robotic devices can assist impaired limbs in performing high-intensity, repetitive, and precisely controlled movements, which helps to restore muscle strength, improve motor function, and enhance overall recovery [3]. These advances highlight the importance of integrating innovative technologies into stroke rehabilitation programs to address the growing burden of long-term disability after stroke.

# Objective of the work [Half page]

- Report the project description that you have as assignment.

- If you have done some further hypothesis (e.g., we suppose that the subject is able to move the hand), outline them here.

This project involves the development of a rehabilitative robotic device for patients in the acute post-stroke phase, aimed at supporting early motor recovery. The target users of this system are individuals with acute post-stroke conditions who present different levels of right upper-limb (right arm) disability. The device is intended to be used in an early rehabilitation setting, where its level of assistance can be adapted to the patient’s residual motor abilities in order to promote active participation and functional recovery.

The core of the project is based on a high-level control strategy that follows an assisted-as-needed approach. Rehabilitation is structured around an interactive brick breaker game, which the patient plays by moving only the impaired right arm to control the game paddle. When the patient is able to correctly reach the area required to catch the ball, the robotic device behaves transparently and does not interfere with the movement. However, when the patient is at risk of losing the ball due to insufficient right arm movement, lack of coordination, or incorrect positioning, the device provides adaptive assistance to guide the arm toward the appropriate region. This strategy ensures that assistance is only provided when necessary, fostering motor learning, patient engagement, and neuroplasticity. The human–robot interface uses an interactive game-based approach to increase patient motivation, promote repetitive right-arm movements, and support safe and effective motor recovery.

Some additional assumptions are made within this project framework. The subject is assumed to be medically stable and cleared by a clinician to participate in robot-assisted right arm rehabilitation. The subject is able to sit safely during the session and must be able to voluntary move the affected right arm, at least a 90° movement without pain. Finally, the patient is also assumed to be able to interact with the robot handle and to possess sufficient cognitive and visual abilities to understand the task and follow the game on the screen.

# Requirements [One page]

## 3.1 Functional requirements: Report functional requirements along with the corresponding means of verification (see LEZ01 for examples).

|  | Description of the requirement | Verification proposal |
| --- | --- | --- |
| Control of the game | The system must allow the patient to control the Brick Breaker game using voluntary movements of the right arm. | At least one user will test the system, and the paddle trajectory will be compared with the measured arm's motion to confirm correct correspondence. |
| ASSISTED-AS-NEEDED CONTROL | The system must implement an assisted-as-needed control strategy, providing assistance only when the patient cannot reach the target independently. | Task execution with and without assistance will be compared. Assistance activation must occur only when performance falls below a predefined threshold. |
| TRANSPARENCY OF THE ROBOT | When the patient performs the movement correctly, the robot must behave in a transparent and non-intrusive manner. | Assistance torque will be measured during correct execution and must remain close to zero. |
| GAME COMPLETION | The system must enable the patient to complete a full Brick Breaker session using only the right arm. | Successful completion of the game session will be verified during testing. |
| REAL-TIME VISUAL FEEDBACK | The system must provide real-time visual feedback of the paddle and ball position. | Direct observation and user feedback will confirm the correct display of real-time information |

## 3.2 Non-functional requirements

Report non-functional requirements along with the corresponding means of verification (see LEZ01 for examples).

|  | Description of the requirement | Verification proposal |
| --- | --- | --- |
| SAFETY | The system must ensure a level of safety, operating within safe torque, velocity, and movement limits. | Torque, velocity, and joint limits will be continuously monitored during training sessions to confirm that values remain within predefined safety thresholds. |
| PHYSIOLOGICAL RANGE OF MOTION | The system must respect the physiological range of motion of the right arm. | Robot joint limits will be compared with standard human ROM values to ensure no constraint or excessive movement is imposed. |
| USABILITY | The system must guarantee high usability, allowing fast setup and operation. | Donning, calibration, and doffing times will be measured and must remain below 15 minutes. |
| USER COMFORT | The system must ensure user comfort, avoiding pain or excessive fatigue during use. | After at least 30 minutes of operation, a user will test the comfort. |
| RELIABILITY | The system must operate without critical failures during repeated training sessions. | Long‑term repeated‑use testing will be performed to verify stable operation without failures. |
| RESPONSIVENESS / PERFORMANCE | The system must provide real‑time responsiveness during interaction. | Interaction will be evaluated to confirm that no noticeable delay is perceived by the user. |

# Materials and methods

## 4.1 Hardware [Half page]

- Include the description of the target of your device (i.e., assistive or rehabilitation target), the type of system, the actuation strategy, portability, target body section(s), target body joint(s), type of assisted human joints, type of implemented robotic joints, and range of motion. Help you out with the contents of LEZ01.

- Describe the hardware included in your project, included all the connections and communication protocol. Include the hardware for the intention detection strategy.

The proposed device is an assistive robotic system for the upper limb, designed as a rigid exoskeleton with revolute joints mechanically aligned to the user’s anatomical joints. It targets elbow and shoulder movements of the right arm, supporting 1 DOF at the elbow (modeled as a hinge joint) and 3 DOF at the shoulder, modeled through three revolute joints that replicate the motion of a ball-and-socket joint. The system operates within a safe range of motion from 0° to 90°, ensuring natural interaction.

The actuation strategy is active and electrical, providing motor-driven assistance during movement execution. The device is stationary, because is it not for portable daily use.

The hardware is based on the EduExo Pro kit, which integrates the mechanical structure, actuators, and joint encoders. The exoskeleton is worn on the user’s right arm and connected to an external PC via USB serial communication, ensuring reliable real-time data exchange. An Arduino board handles low-level hardware communication and sensor acquisition.

The game interface is developed in Unity, while control logic is implemented in Visual Studio. User intention is detected through kinematic data from the joint encoders, allowing the system to estimate voluntary movement and modulate assistance according to an assisted-as-needed strategy.

## 4.2 Kinematic and dynamic model [One page]

Describe the kinematic and dynamic model. Refer to LEZ02 and LEZ03. Describe the strategy used to define the reference trajectory (if any) or the reference torque profile (if any). See LEZ05 for reference. Calculate the relevant relationships between joint and end-effector coordinates. (When you delete the dark sentence it fit one page)

The proposed device is a 2 DOF upper limb exoskeleton supporting shoulder and elbow flexion‑extension. Its kinematic model is based on a planar representation of the right arm, where the end‑effector position depends on the joint angles 𝜃  _1 (shoulder) and 𝜃 _2 (elbow), and on the segment lengths L_1 and L_2. The forward kinematics is expressed as:

𝑥𝐸𝐸=𝐿1cos𝜃1+𝐿2cos𝜃1+𝜃2 1, 𝑦𝐸𝐸=𝐿1sin𝜃1+𝐿2sin𝜃1+𝜃22

This formulation allows real‑time estimation of the user’s arm position during the Brick Breaker task. Inverse kinematics is solved numerically (pseudo‑inverse or damped least squares), enabling the system to compute the joint configuration required to reach a desired target.

The dynamic model follows the standard rigid‑body formulation: 𝜏=𝑀𝜃𝜃⋅⋅+𝐶𝜃,𝜃𝜃⋅⋅+𝐺𝜃 3. Where (M) inertia matrix, (C) Coriolis and centrifugal effects, and (G) gravity. Joint angles measured by the EduExo Pro encoders allow numerical estimation of velocities and accelerations, enabling torque modulation according to the assisted‑as‑needed strategy. Consistent with LEZ03, the device uses stiff joints actuated by electric motors, and compliance is ensured through low‑level torque control and safe velocity limits, promoting transparent interaction.

The proposed system uses a reference trajectory to guide the movement of the end‑effector (the paddle in the Brick Breaker game). The trajectory is defined in Cartesian space and then mapped to joint space through the robot’s kinematic model. Since human arm movements naturally follow smooth profiles that minimize energy expenditure, the reference trajectory is generated using a minimum‑jerk model, which ensures smooth, physiologically plausible motion. The minimum‑jerk trajectory is expressed as a fifth‑order polynomial:

𝑥𝑡=𝑎0+𝑎1𝑡+𝑎2𝑡2+𝑎3𝑡3+𝑎4𝑡4+𝑎5𝑡5 4

The coefficients are computed from boundary conditions on position, velocity, and acceleration at the start and end of the movement. This trajectory defines the desired paddle motion between two target points in the game. To convert the Cartesian reference into joint commands, the forward kinematics of the 2‑DOF arm model are used equation (1) and (2).

These relationships allow computing the desired joint angles through inverse kinematics, ensuring that the exoskeleton follows the planned trajectory. In the assisted‑as‑needed mode, the reference trajectory is used only when the user cannot reach the target independently; otherwise, the robot behaves transparently, providing no corrective torque. In case of torque‑based control, the reference torque profile is derived from the dynamic model, ensuring that assistance is applied only when the user’s generated torque is insufficient.

## 4.3 Control design [Three pages]

### Hardware level of compliance (LEZ03)

The device uses stiff joints actuated by electric motors with gearbox transmission, which classify the system within the stiff‑actuation category. This type of hardware does not provide intrinsic mechanical compliance, meaning that all compliant behavior must be generated through control rather than through elastic elements such as SEAs or cable‑driven mechanisms.

The exoskeleton integrates joint encoders for position sensing, enabling joint‑space feedback but not direct measurement of interaction forces. Since no torque sensors or series elastic elements are present, the system relies on indirect estimation of interaction forces through motor current and kinematic deviations.

To ensure safe and transparent human‑robot interaction, the low‑level controller implements torque limiting, velocity limiting, and soft virtual boundaries, which emulate compliant behavior despite the stiff hardware. These mechanisms reduce the effective stiffness perceived by the user and prevent excessive forces during unexpected movements or collisions.

The absence of mechanical compliance requires the controller to regulate assistance through low‑level torque modulation, ensuring that the robot behaves transparently when the user moves correctly and provides support only when needed. This approach is consistent with hardware‑level compliance strategies for stiff‑actuated exoskeletons, where compliance is achieved through control rather than physical elasticity.

### Low-level control solution (LEZ04) (Help... not finished)

The low-level control architecture is designed to ensure compliant and safe interaction between the user and the exoskeleton, despite the stiff and non‑backdrivable nature of the hardware. Since the EduExo Pro does not include torque sensors or series elastic elements, compliance must be generated entirely through control rather than through mechanical properties. For this reason, the low-level control strategy adopted is an explicit impedance control strategy implemented in joint space.

The controller regulates the relationship between joint position errors and motor torque, making the robot behave like a virtual mass spring damper system. This ensures a balance between tracking accuracy and compliant interaction, preventing excessive forces on the user. The reference torque is computed from the deviation between the measured joint angle and the desired trajectory:

𝜏imp=𝐾𝑠𝜃ref−𝜃𝑚+𝐾𝑑𝜃ref−𝜃𝑚

......

### High-level control solution (LEZ05)

### Intention detection strategy (LEZ06)

## 4.4 Protocol for testing and validation (LEZ07) [One page]

Describe the protocol and the correspondent outcome measure(s) to test your system:

- Component test protocol: describe the tests and the correspondent outcome measure(s) that you perform to validate the single modules.

- System test protocol: describe the tests and the correspondent outcome measure(s) that you perform to validate the correct functioning of the complete system.

Describe the experimental procedure/protocol you used for each result that you report.

Be careful that you need to verify the requirements previously outlined.

# Results [Three pages]

Report the results obtained in your project. You need to report the results correspondent to all methods you described before (section 4.4).

When reporting data, please make sure that:

- you have done more than one repetition, and therefore to report mean and standard deviation or median and interquartile range;

- you report all units of measurements;

- you make proper (parametric/non parametric) statistics if relevant;

- you report numbers to support the conclusion that you have met/not met the requirements.

Please note that if you don’t meet a requirement, you can justify your result -- it is not a failure.

Please make use of tables and figures to support your findings. Prepare your figures within a SW (i.e., do not stamp Arduino screen or similar).

# Discussion and conclusion [One page]

Discuss the obtained results, particularly with respect to the initial objectives you have. Outline limitations and possible future works.

# References [Half page]

[1] Feigin VL, Brainin M, Norrving B, Martins SO, Pandian J, Lindsay P, F Grupper M, Rautalin I. World Stroke Organization: Global Stroke Fact Sheet 2025. Int J Stroke. 2025 Feb;20(2):132-144. doi: 10.1177/17474930241308142. Epub 2025 Jan 3. PMID: 39635884; PMCID: PMC11786524.

[2] Azzollini, V., Dalise, S., & Chisari, C. (2021). How Does Stroke Affect Skeletal Muscle? State of the Art and Rehabilitation Perspective. Frontiers In Neurology, 12, 797559. https://doi.org/10.3389/fneur.2021.797559

[3] What to expect as you recover from a stroke. (s. f.). Mayo Clinic. https://www.mayoclinic.org/diseases-conditions/stroke/in-depth/stroke-rehabilitation/art-20045172

[4] Ambrosini, E., & Gandolla, M. (2025). Collaborative Robotics (Section A) [Course materials]. Politecnico di Milano.

[5]